User & App Functions

- The system shall allow users to create an account and log in to the mobile application.

- The system shall allow users to register a SmartFix Box to their account.

- The system shall display the online or offline status of the SmartFix Box in real time.

- The system shall allow users to view a list of monitored smart devices.

- The system shall allow users to assign friendly names to monitored devices.

- The system shall allow users to manually restart a selected smart device from the app.

- The system shall display system logs and recent actions performed by the SmartFix Box.

- The system shall send notifications to the user when a device goes offline.

- The system shall notify the user if the SmartFix Box itself goes offline.

- The system shall allow users to configure alert preferences (push, email, frequency).

SmartFix Box (Raspberry Pi) Functions

- The system shall automatically boot and start monitoring services on power-up.

- The system shall scan the local network to detect compatible SmartFix smart plugs.

- The system shall continuously monitor the connectivity status of registered devices.

- The system shall detect when a monitored device becomes unresponsive.

- The system shall attempt an automatic restart of a device when a failure is detected.

- The system shall limit restart attempts to a configurable threshold.

- The system shall log all detected outages and recovery actions.

- The system shall expose a local REST API for communication with the mobile app.

- The system shall securely authenticate API requests from authorized applications.

Networking & Setup

- The system shall allow initial network configuration through the mobile application.

- The system shall store network credentials securely on the device.

- The system shall automatically reconnect to the network after a power or network outage.

- The system shall support operation without continuous internet access.

- The system shall synchronize status data with the cloud when connectivity is restored.

Cloud & Synchronization

- The system shall synchronize device status and logs with a cloud service.

- The system shall allow the mobile app to retrieve the last known device status from the cloud.

- The system shall support multiple SmartFix Boxes under a single user account.

- The system shall coordinate multiple SmartFix Boxes operating on the same network.

Security & Reliability

- The system shall encrypt all communications between the SmartFix Box, mobile app, and cloud services.

- The system shall restrict device control actions to authenticated users only.

- The system shall continue monitoring devices in the event of temporary cloud failure.

- The system shall recover gracefully from unexpected restarts or crashes.